Except from Northwatch Submission on Whiteshell Laboratory Relicensing 2019

### CNL’s Proposed Use of NWMO Used Fuel Transportation Package

We noted with interest CNL’s stated intentions to use the Nuclear Waste Management Organization’s (NWMO) Used Fuel Transportation Package (UFTP):

The fuel baskets will be retrieved from the canisters (see Figure 3-3) and transferred to the

Used Fuel Transportation Package (UFTP) (see Figure 3-4 and Figure 3-5), for transport to and storage at CRL. The UFTP is a CNSC-certified Type B(U) Transportation Package, leased by CNL from its owner, the Nuclear Waste Management Organization (NWMO), for transporting CNL fuels, including the WL fuel materials. The UFTP is undergoing a comprehensive licensing process for CNL-specific fuels and configurations. Concrete canisters to contain the WL spent fuel baskets are being constructed at CRL. CNL will remain in communication with CNSC staff at all stages of this process, and regulatory oversight by CNSC staff will remain in effect.

As described in CNL’s CMD, nuclear fuel currently on site at Whiteshell – which CNL intends to transport using the NWMO’s UFTP, includes both intact, irradiated fuel bundles and  sealed storage cans of defective fuel and fuel fragments.

The NWMO’s Used Fuel Transportation package was developed by the NWMO as a reference transportation package, and used by the NWMO for such purposes as conducting “generic” assessments of radiation dose for use in report being produced as part of their “Adaptive Phased Management” program.

The UFTP was first certified in the 1980’s as a contribution by Ontario Power Generation (then Ontario Hydro) to Atomic Energy of Canada Limited Geological Disposal Concept. In 2013, the UFTP was recertified by the Canadian Nuclear Safety Commission staff, without public review.

When the CNSC issues a certificate for the package design, the certificate specifies procedures for the manufacture, operation and maintenance of the transportation package. It also defines the authorized contents that may be carried in the package. The certificate is valid for five years.  As set out in the certificate issued by the CNSC in 2013, the UTFP is designed for intact fuel bundles. The UTFP was recertified in 2018.

This intended use of the NWMO’s UTFP by CNL raises two questions immediately:

- Given that the UFTP has been certified for intact fuel bundles and the CNL high level radioactive wastes which they have indicated they intend to ship to Chalk River includes fuel waste which is defective and /or is fuel fragments, what is the basis for selecting this UTFP?

- CNL states that the “UFTP is undergoing a comprehensive licensing process for CNL-specific fuels and configurations’: what is the nature of that comprehensive licencing process, and what oversight is being provided by the Commission and what are the opportunities for review by the interested and potentially impacted public, First Nations, and en route communities?

Northwatch would note that this appears to be another instance of mission creep on the part of the NWMO, although in the absence of full disclosure of related information, it is difficult to ascertain the degree or implications of this.

On a somewhat more humorous note, we appreciated CNL’s selection of a photo of NWMO’s mock-up of their Used Transportation Fuel Package, perhaps as in indication of their degree of being “road ready”.  The selected photo is of a transportation exhibit used for promotional purposes by the NWMO when visiting municipalities who have encouraged the NWMO to study areas in their vicinity as potential burial sites for all of Canada’s high level nuclear fuel waste.

CMD 19-H4.1 Page 21

CMD 19-H4.1 Page 20

NWMO TR-2014-17 December 2014, Generic Transportation Worker Dose Assessment

Safe and Secure Transportation of Canada’s Used Nuclear Fuel MAY 2015 NWMO, page 14

Canadian Nuclear Safety Commission (CNSC). 2013. Certificate for Transport Package

Design. CDN/2052/B(U)-96 (Rev. 7). CNSC File 30-H1-118-0. July 29, 2013.

CMD 19-H4.1 Page 23, “Figure 3-5: Used Fuel Transportation Package for the removal of CCSF fuel to CRL”

See, for example, https://www.nwmo.ca/en/More-information/News-and-Activities/2017/10/06/15/25/Transportation-Exhibit-Attracts-Hornepayne-Students or https://www.nwmo.ca/en/More-information/News-and-Activities/2016/10/04/12/54/Used-Fuel-Transportation-Package-on-Display-at-Lucknow-Fall-Fair or https://clcinfo.ca/hornepayne/files/2013/06/Jackfish-Journal-NWMO-Transportation-Exhibit-at-FONOM-Conference-May2013.pdf